Router Guide — Secure Syslog over TLS to the New Server

Audience: the network / router administrator.

Goal: configure a Cisco router to send its syslog securely — TLS over TCP, port 6514 (RFC 5425) — to the migrated syslog server.

The server now requires encrypted, mutually-authenticated syslog. Plaintext UDP 514 is being retired. This guide walks the router side end to end. Exact CLI varies by IOS / IOS-XE train — confirm against your version.

# 1. Server endpoint

Point the router at:

| Setting | Value |
| --- | --- |
| Server FQDN | <SERVER-FQDN> — must resolve; the router validates the server certificate by name |
| Server IP | <SERVER-IP> |
| Port / transport | 6514 / TCP |
| TLS version | 1.2 or higher |
| Authentication | Mutual TLS — the router must present its own client certificate (Step 2) |
| Trust anchor | DoD PKI — the same CA chain that signed the server certificate |

Fill in <SERVER-FQDN> and <SERVER-IP> from the server owner before you begin.

# 2. Prerequisites (on the router)

Confirm these first — TLS fails cryptically if any are wrong:

- Accurate clock. The router must be NTP-synced. Certificate validation rejects an out-of-tolerance clock.

- Name resolution. The server FQDN must resolve (DNS, or a static ip host entry). The name must match the server certificate or validation fails.

- Reachability. TCP 6514 to <SERVER-IP> must be permitted outbound, through any ACL or firewall in the path.

# 3. Step 1 — Trust the syslog server’s CA

Install the DoD CA chain so the router trusts the server’s certificate.

crypto pki trustpoint SYSLOG-CA

enrollment terminal

revocation-check none

crypto pki authenticate SYSLOG-CA

<paste the CA / root certificate(s) in PEM, then type: quit>

If the chain has intermediates, authenticate the full chain (root plus each intermediate) so the router can build a path to the server certificate.

# 4. Step 2 — Enroll a router identity certificate (mutual TLS)

The server verifies the router, so the router must present its own certificate. Enroll one from the same DoD PKI, with the router’s FQDN as the subject.

crypto pki trustpoint ROUTER-ID

enrollment <your PKI enrollment method>

subject-name CN=<router-fqdn>

rsakeypair ROUTER-ID 2048

crypto pki authenticate ROUTER-ID

crypto pki enroll ROUTER-ID

Record the resulting certificate’s Subject (e.g. CN=<router-fqdn>) — the server owner needs it to authorize this router (Step 4).

# 5. Step 3 — Configure TLS syslog

logging tls-profile SYSLOG-TLS

tls-version 1.2

trustpoint client ROUTER-ID

!

logging host <SERVER-IP> transport tls port 6514 profile SYSLOG-TLS

logging source-interface <mgmt-interface>

logging trap informational

Older IOS without logging tls-profile: use logging host <SERVER-IP> transport tls port 6514 and bind the CA and identity trustpoints per that train’s syntax.

Set logging trap to the severity your site requires; informational captures most operational events.

# 6. Step 4 — Send two items to the syslog server owner

The server pins which devices may connect, so before the router can be accepted it needs from you:

- 1.  The router’s client-certificate Subject / SAN (e.g. CN=<router-fqdn>) — added to the server’s allow-list.

- 2.  The source IP the router will send from — for the server’s firewall.

Until the server owner adds your Subject to the allow-list, the router’s connection is refused by design. That is the mutual-TLS check working, not a fault. Coordinate the cutover so both sides are ready.

# 7. Step 5 — Verify

On the router:

show logging                        ! the TLS host should show, state active

show crypto pki trustpoints status  ! both trustpoints should read "Valid"

The server owner confirms receipt on their side (established connection on 6514, and messages landing in the log files). Send a test event — e.g. a config change or an interface flap — and confirm it arrives.

# 8. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Connection refused / reset | Router not yet in the server allow-list, or firewall closed | Confirm the server owner added your cert Subject (Step 4) and opened 6514 to your source IP |
| Certificate validation failed | Router clock off, missing CA/intermediate, or FQDN mismatch | Check NTP, re-authenticate the full CA chain (Step 1), confirm the server name resolves and matches its cert |
| No matching cipher / handshake failure | TLS version or cipher mismatch | Confirm tls-version 1.2+; check the server’s allowed ciphers with its owner |
| Nothing arrives, no errors | logging trap too low, or wrong source interface / route | Raise logging trap, verify logging source-interface and the route to <SERVER-IP> |

# 9. If client certificates are not feasible

If this router cannot be issued an identity certificate, tell the server owner. They can switch the collector to server-authentication only — the session is still encrypted in transit, but the server no longer verifies the router by certificate. In that mode:

- Do Step 1 only (trust the server CA).

- Skip Step 2 and omit the trustpoint client line in Step 3.

Mutual TLS is stronger and preferred; use this only when identity-cert enrollment is not possible.

Questions on the server side (endpoint, allow-listing, cipher policy) go to the syslog server owner.